# torching and welding

When doing torching or welding activities on a commercial roof, employees should have working knowledge about equipment, housekeeping practices, as well as fire prevention.

Torching and welding equipment:

- Ensure safety relief valves on cylinders remain unobstructed

- When not in use a safety plug should be in place for each cylinder valve

- Inspect all compressed cylinders for defects before use and always secure to prevent tipping

This Photo by Unknown Author is licensed under CC BY

Place torch and hand held welders on stand when not in use

- Inspect all hoses to check for any wear or tearing before use

- A regulator should be used any time torch or welding equipment is being used and all connectors should be leak tested before use

- Light torches with a spark lighter not with cigarette lighters or matches

- Do not leave torch connected to the cylinder when finished using

- Use walk behind welders when possible over handheld to reduce strain

Fire prevention:

- Know the expectations of the company emergency preparedness plan and follow them if a fire occurs

This Photo by Unknown Author is licensed under CC BY-NC-ND

All rooftop mechanical equipment should be disconnected and removed prior to these activities

- A charged fire extinguisher should be within easy access during these activities

- Good housekeeping practices should be maintained on the jobsite with debris and flammables removed before beginning

This Photo by Unknown Author is licensed under CC BY-NC-ND

Natural gas service lines should be removed prior to torching; be alert to natural gas odor

- Shut down all rooftop mechanical units and remove filters as they pose a fire hazard

- Only torch areas that can be seen and never leave a torch lit unattended

- Gussets should be applied at internal and external corners to prevent flame penetration

Protective clothing:

- Long sleeve shirts and pants as well as work boots should be worn made from non-synthetic materials; clothing should be made of wool or cotton

- Wear leather work gloves with sleeves overlapping them

- Safety glasses should be worn as well