Sample Safety Manual

Steel Erection

# Insert Company Name

# SAFETY POLICY STATEMENT

As a (insert Company Name) employee, you are a valuable asset to the Company. The policy of (insert Company Name) is to provide a working environment free of uncontrolled hazards in support of its goal in providing a safe and healthy workplace. We want, and expect, you to perform your job in a safe manner in response to the circumstances that may arise during the course of your employment.

# INTRODUCTION

This manual contains safety policies developed to control the risks associated with Company operations. It is understood that it is impossible to attempt to write a rule for every future work scenario and, therefore, while many policies are detailed in this manual, others dealing with your specific job responsibilities may be given to you by your supervisor or their designee specific to the recognized job tasks at hand.  This manual has been designed for a single purpose; to assist you in avoiding workplace injuries or illness.

If you have a suggestion that could reduce the potential for accidents in your work operations and would make your place of employment even safer, please talk with your supervisor or supervisor designee.  Please remember that at (insert Company Name), accident prevention is a part of every employee’s job.

Accidents are the result of unsafe conditions and/or unsafe acts or practices.  Many accidents are caused by the use of unsafe equipment, using tools in an unsafe manner, or by neglecting to follow the safe methods of doing your job.

Please note that violations of Company safety policies or unsafe practices could result in severe and painful injury, as well as termination of employment, as compliance with Company safety policies is a condition of employment. Failure to follow either established Company policies or supervisor’s directions subject the employee to the Company’s disciplinary procedures.

Questions and requests for additional information relative to the contents of this manual should be directed to your immediate supervisor/manager/foreman or their designee.

# SAFETY RESPONSIBILITY

- MANAGEMENT RESPONSIBILITIES

Every employee of (insert Company Name) is responsible for their own personal safety. This begins with Company management and extends throughout the organization. Company management is responsible for providing a safe work site and adequate supervision of its operations. This includes, as applicable, the provision of tools and equipment, proper safety equipment/clothing, training and on-site direction. In addition, management is responsible for implementing the following:

- Periodic inspections.

- Ensure that prompt preventative and corrective action is taken on unsafe conditions/actions.

- Ensure that all accidents are investigated and reported.

- Review reports of accidents and ensure appropriate corrective actions are taken.

- SUPERVISOR’S RESPONSIBILITIES

- Promote safety awareness and demonstrate a proper safety attitude by example.

- Train all employees in the safest way to do their jobs and point out where hazards exist.

- Make sure that the necessary safety equipment and protective devices for each job are provided and properly used.

- Conduct frequent safety inspections of all work areas and operations in order to improve housekeeping and eliminate unsafe conditions while encouraging safe work methods.

- Take prompt corrective action whenever unsafe conditions and unsafe actions are observed.

- Investigate and report to management all accidents and incidents involving Company personnel and/or property.

- Review work change practices and newly purchased equipment for potential risks being introduced into the operations.

- Inform employees regarding disciplinary policy for violations of safety policies and take appropriate action when warranted.

- EMPLOYEE’S RESPONSIBILITIES

- Follow Company safety practices, policies, procedures and specific supervisor instructions.

- Report unsafe conditions and practices to the supervisor.

- Keep work areas clean and orderly at all times.

- Operate only equipment you have been authorized and instructed to safely use.

- Report all accidents/incidents immediately to the supervisor and complete an accident report as soon as possible.

##### IF YOU HAVE AN ACCIDENT

- All accidents/incidents, no matter how minor, shall be reported immediately to your supervisor. Prompt reporting will ensure proper treatment (if required) and a rapid correction of unsafe conditions or behavior causing or contributing to the accident.

- The supervisor will arrange for transportation and accompany the injured employee to the medical facility to facilitate paperwork and address questions by both the treating physician and injured/ill employee.

- The supervisor and employee must complete the Company’s accident report forms as soon as possible and route as directed.

- First aid and CPR shall be performed only by trained personnel.

- All employees will follow established Bloodborne Pathogens procedures.

# EMERGENCY ACTION PLANS

Emergencies (fire, severe storms, bomb threats, etc.) require immediate action by all employees and visitors.  The following emergency action plans have been developed with supervisors/managers or their designee responsible for monitoring and overseeing the evaluation/activities.

Supervisors/Managers or designee for their respective areas will: direct actions of personnel; be familiar with the location of fire extinguishers; be aware of individuals who have first aid or CPR training; be aware of individuals that will need special assistance during building evacuation and make arrangements to assure their safety; familiarize employees with evacuation route; ensure their building area is clear and account for personnel after building evacuation.

- Fire Evacuation

- When the fire alarm sounds, turn off all equipment and walk to the nearest stairway/exit out of the building (do not use elevator).

- Assemble away from the building at the manager’s designated area.

- Assist persons with special needs to designated areas (persons unable to use stairs should wait in the stairwells for Fire Department rescue).

- Managers or their designee will account for all individuals from their areas. Employees will remain outside until the all clear is given.

- Severe Storms – Tornadoes

In severe weather, you are safer in the building than in the open and, therefore, you should not leave the building during these events. If the need arises, you should move to the designated storm shelter in your work area.

- Bomb Threats

While some bomb threats are hoaxes, each one must be treated seriously. The receiver of the bomb threat should attempt to obtain as much information as possible regarding the threat. The incident should be reported immediately to your supervisor Evacuation procedures are the same as a fire evacuation.

# VIOLENCE IN THE WORKPLACE

Violence in the workplace by customers and/or co-workers is becoming more common. For this reason, it is the policy of (enter Company name) that physical violence, or the threat of violence, even in a joking manner, is not appropriate in the workplace and may result in disciplinary action.

Company employees are to report all violence or threats of violence to their supervisor and/or manager at the earliest opportunity. Report the actual behavior or threats that were made, give the facts of where and when it happened, who witnessed it, and what was said.

Any employee who observes anyone with a weapon on the premises shall report the observation immediately to their supervisor and/or manager at the earliest opportunity.

BIOLOGICAL EXPOSURE CONTROL PLAN

During the course and scope of your job, you may come into contact with co-workers, customers and/or members of the public who pose a risk of communicating blood and/or airborne diseases. For your protection, do not allow yourself to come into unprotected contact with co-worker’s blood or other body fluids. See Blood/Airborne Pathogens Program

## ERGONOMICS

Depending on the physical demands of your job, changes in the physical layout or actual work procedures may be required to minimize your risk of cumulative trauma type injury. These changes can only come about as a result of your communication of any work-related conditions that can be resolved through an ergonomic evaluation of your work area/practices. At that time, modifications will be made, as appropriate, with the understanding that you will support these changes.

## RECORDKEEPING

Documentation is a critical component of any effective safety effort. As such, the following records will be develop and maintained in Company files:

- Employee training records

- Certifications

- Accident/Incident reports

- Hazard identification and correction actions

- Signed acknowledgements

- DOT required records

- Equipment inspection/certifications

- OSHA 300, 300A, 301

# HAZARD COMMUNICATION

All potentially harmful products used by the Company are to be evaluated and the risks associated with these products communicated to the affected employees prior to use and/or exposure to these products. Annual training on this subject will be provided with additional training as new products are introduced in the work area. See Hazard Communication Program

# EXTREME TEMPERATURE CONDITIONS

Company operations may expose employees to temperature extremes in the workplace. These exposures may result in possible injury or illness and, as such, the Company has developed procedures to address and control these situations.  Employees are responsible for monitoring their own physical condition both on and off the job as they relate to their ability to tolerate these conditions. See Temperature Extremes Program

# GENERAL POLICIES FOR ALL OPERATIONS

You should be familiar with and practice safe methods of doing your job. If in doubt, ask your supervisor!

Illness and/or extreme fatigue may affect your ability to perform your job safely. You are responsible for monitoring your personal physical condition and taking appropriate actions to ensure that you receive adequate rest and that medications do not impact your job performance.

Horseplay of any kind is strictly forbidden and employees who willfully engage in horseplay are subject to the Company’s disciplinary procedures up to, and including, termination.

Clean and orderly work areas and equipment are a requirement for both accident and fire prevention. Take the time to keep your work area clear of unnecessary materials, tools and/or equipment.

Keep a three-foot radius area around fire extinguishers and electrical panels clear of materials and stored equipment. In an emergency situation, this equipment needs to be readily accessible.

All exits and stairways should be maintained clear and free of storage and other obstructions.

Only approved, grounded electrical extension cords shall be used and only when fixed electrical service is not available. All extension cords, when not in use, must be unplugged, coiled and stored in their designated locations.

Lifting and other material handling shall be performed using proper lifting techniques to include the assistance of other personnel as necessary. When using lifting equipment (hoists, cranes, etc.), the equipment will be checked for proper condition/operation prior to actual loading.

The use of personal music device (IPOD, MP3, radio) or other sound producing devices in the work area (to include vehicles) will be at the discretion of the immediate supervisor. At no time, however, will any device volume be in excess of 85 decibels.

Smoking is prohibited outside of designated “Smoking Areas.”

Make visual checks for unsafe conditions before starting any machinery or equipment. Make all daily operator checks before starting machinery or equipment.

Do not start equipment/machines without a visual check to ensure work area is clear.

Do not leave any machines or equipment running while unattended.

Loose clothing, long hair and/or jewelry shall not be worn by personnel who work around or near moving machinery or equipment.

All guards must be in place before and while operating the equipment/machinery.

Appropriate Lockout/Tagout procedures will be followed prior to any repair work being performed on any machinery and/or equipment.

# PERSONAL PROTECTIVE EQUIPMENT

# Eye Protection

Appropriate safety glasses, goggles, or face shields shall be worn when the work may result in a potential risk of injury to your face/eyes. Safety glasses and goggles are provided by the Company and employees are responsible for keeping track of and maintaining the equipment in good condition. Damaged equipment must be replaced immediately.

Some work examples where proper eye protection must be worn include exposure to:

- Relatively large flying particles (chipping, grinding, etc.).

- Dust and small flying particles (grinding, using power tools, etc.).

- Splashing liquid (transfer of products, etc.).

- Injurious sprays and mist (spraying, use of products, etc.).

- Injurious radiant energy (welding, furnace tending, etc.).

# Head Protection

Approved head protection (hard hats) must be worn when the work may result in a potential risk of injury to your head. Head protection must be worn as recommended by the manufacturer to include replacement by the stamped date. Employees are responsible for keeping track of and maintaining this equipment in good condition.

# Hearing Protection

The Company has elected to implement a Hearing Conservation Program to control the risks associated with high noise levels in the work area. Approved ear protection shall be worn under all work conditions that have been determined by the Company to exceed acceptable noise levels. Hearing protection shall be provided by the Company and employees are responsible for keeping track of and maintaining the equipment in good condition. See Hearing Conservation Program

Respiratory Protection

The Company has elected to implement a Respiratory Protection Program to control the risks associated with workplace exposure to dusts, vapors, fumes and/or other airborne particulates. Approved respirators shall be worn when working in or near areas designated by the Company as potentially hazardous due to inhalation exposures. Employees are responsible for keeping track of and maintaining the equipment in good condition. See Respiratory Protection Program

Foot and Hand Protection

Shoes or boots suitable to the type of work and work area conditions shall be worn at all times. Slip resistant soles may be required in some operations. Safety-toed footwear may be required in those situations where a higher-level risk of foot injury exists. Open toe shoes are prohibited from all work areas outside of offices.

Hand protection will be chosen based on the risk potential and utilized by the employee during the performance of their assigned work activities. Consult your supervisor in regard to glove selection and use.

Outerwear  Protection

Regular work clothing will be cotton or other natural fiber. Shorts and tank top shirts are prohibited. Where required, the Company will provide splash aprons or Tyvek coveralls. Employees are responsible for keeping track of and maintaining their clothing and Company provided gear in good condition or replaced as necessary. Contaminated clothing or gear will be removed and replaced when the contaminants pose a health or fire hazard.

FALL PROTECTION

Work on raised platforms, ladders or other elevated work locations creates the risk of serious injury. Appropriate fall protection in the form of railings, barriers or personal fall protection harnesses will be used anytime work is performed above normal grade, finished floor, or over water. See Fall Protection Program

SCAFFOLDING

Scaffolds are a major source of injuries and fatalities. The (Company) will evaluate all potential scaffolding hazards/deficiencies at job sites. A competent person will evaluate and communicate information concerning these hazards. Established appropriate procedures, and protective measures for employees will be followed. See scaffolding program.

# LADDERS

Do not use chairs, boxes or other objects in lieu of approved ladders.

Ladders must be regularly inspected before use. All defective ladders (weakened, broken or missing steps, broken side rails, etc.), must be tagged, removed from service and reported to the supervisor immediately. All portable ladders shall have non-skid bases.

In placing an extension ladder, the ladder should be one foot out for every 4 feet up and the ladder should extend at least 3 feet above the roofline and be tied off at the top.

When using a step-ladder longer than ten feet high, another person must hold the ladder.

Use both hands when ascending and descending ladders and always face the ladder.

Only one employee is to use a ladder at any time.

Do not climb higher than the 2nd step from the top of a step-ladder. Do not climb straight ladders higher than the third step from the top.

Do not use metal ladders near energized electrical circuits.

OPERATION OF CRANES

Cranes are carefully designed, tested, and manufactured for safe operations. When used properly they can provide safe reliable service to lift or move loads. Only qualified and properly designated people shall operate a crane for this company. Operators should be familiar with rated load limits and recommended operating speeds, special hazard warnings, or instructions on all equipment. Before operations begin for the day, an inspection needs to be conducted to ensure that the machine is in proper working condition. Added requirements are included in the Crane Safety Program.

FORKLIFTS, AERIAL, SCISSOR LIFTS

Due to the risks associated with working on and around forklifts, aerial lifts, and scissor lifts, the Company has implemented programs that includes the following requirements:

- Only authorized and certified personnel shall operate lift trucks.

- Operators are responsible for attentive driving and personal awareness of their surroundings and possible changes in the work area. Seatbelts shall be worn by the forklift operator at all times while operating the equipment.

Additional requirements involving forklift operations are included in the Company’s Forklift Program and Aerial and Scissor Lift Program.

RIGGING

Because the possibility of serious accidents resulting in personnel injury, death or significant property damage exists whenever hoisting and rigging take place, the requirements for these operations must be clearly defined and precautions ensured, including proper preplanning, extreme care, attention to detail, teamwork on the part of trained operators/riggers, and the use of equipment that is reliable, properly designed, inspected, and maintained. Anyone designated as a rigger must be trained and certified by a qualified person in the use of the equipment and authorized to perform rigging. Full requirements can be found in the written program for rigging.

SIGNALING

A crane operator should always move loads according to the established code of signals and use a signaler that is in direct view of the operator at all times. Signalers shall be qualified and trained in the standard hand signals. See written program for hand signalers.

LOCK OUT/TAG OUT

Due to the serious risk of injury and possible death, this policy establishes the requirements necessary to safeguard employees while performing, servicing, or maintenance tasks, by the removal of energy sources (electrical, hydraulic, pneumatic, chemical, thermal and kinetic) that may cause serious injury to employees or property damage by accidental start-up of machinery, equipment, or processes. See Lock Out/Tag Out Program.

HAND TOOLS

Hand tools are to be used only for the purpose for which they are designed. They will be inspected before use and replaced as required.

Tools with burrs, cracks, mushroom heads, or broken, loose or splintered handles shall not be used.  Turn them in for repair or replacement.

Do not leave tools on overhead work areas or areas where they pose a potential risk of injury or where they will fall and strike someone working below.

Never leave tools lying around where they may be tripped over or interfere with other work in progress.

Do not carry an edged or pointed tool in pockets or belts unless the point or edge is protected.

PORTABLE POWER TOOLS

Never use any portable power tools without checking for both proper safeguard operation, (e.g. safety on pneumatic nail gun and spring loaded guard on skill saw) and electrical continuity. Questionable items should be inspected and tested by qualified personnel.  Always report defects or minor shocks on any piece of equipment. Any electrical equipment or cord without a ground prong intact shall not be used.

Ground fault circuit interrupters (GFCIs) shall be used in all wet locations.

Electrical cords should be protected from damage by oil or wear and not left in aisles where they may be run over by trucks or other equipment or cause a tripping hazard. Never run an electrical cord through a doorway.

WELDING OPERATIONS

Wear the proper clothing (preferably wool as it will protect the body from rays of the arc produced from metal sparks). Shoes should extend above the ankle and leg coverings should extend below the tops of the shoes. Trousers should not have cuffs.

Hoods must be in place before you strike an arc and at all times while welding.

Welder’s helpers shall be protected in the same manner when in the immediate work area.

Shields must be in place to protect other employees from the rays of the arc.

All hot work procedures must be followed. See Hot work permit program

CUTTING OPERATIONS

Torches, hoses and gauges will be maintained in proper working order.

Gas cylinders must be kept in racks or stands, or set in an upright position and properly secured to prevent their being knocked over. They should be transported in an upright position with regulators removed and cylinder caps in place.

Never use LP pressure above 15 psig. LP pressures above 15 psig can result in a fire or explosion.

Do not handle oxygen regulators, oxygen cylinders, valves, or any other equipment with oily or greasy hands or gloves. Oxygen will react with oil and grease in such a manner that may result in a fire or explosion.

Never discharge fuel gas from a cylinder valve near welding or cutting work, sparks, flame or any other possible source of ignition.

Always release pressure on adjusting screw before opening cylinder valve.

Never loosen a connection or attempt to remove a part until gas pressure has been relieved. Under pressure, gas can dangerously propel a loose part.

Open cylinder valve SLOWLY. The oxygen cylinder valve should be opened completely, acetylene cylinder valve should be opened ½ to 1 turn, and other fuel gas cylinders should be opened completely.

Keep cylinder hand wheel or wrench on open cylinder valve at all times, for prompt emergency shutoff.

All hot work procedures must be followed. See Hot work permit program

COMPRESSES GAS CYLINDERS

Gas cylinders must be kept in racks or stands, or set in an upright position and properly secured to prevent their being knocked over. They should be transported & stored in an upright position with regulators removed and cylinder caps in place.

Do not handle oxygen regulators, oxygen cylinders, valves, or any other equipment with oily or greasy hands or gloves. Oxygen will react with oil and grease in such a manner that may result in a fire or explosion.

Never loosen a connection or attempt to remove a part until gas pressure has been relieved. Under pressure, gas can dangerously propel a loose part.

Areas in which compressed gases are used must be properly ventilated. The type and degree of ventilation depends upon the type of gases being used.

Gas cylinders must never be dropped or treated roughly.

MATERIALS HANDLING AND STORAGE

Employees shall evaluate the object(s) to be lifted prior to attempting the lift. Get help or divide the load and always utilize proper body mechanics when lifting.

Lifting aids such as portable cranes, hand trucks, skids, hoists or power lift trucks should be used to move heavy objects whenever possible.

Work gloves should be worn when handling heavy or rough objects.  Foot protection is required when there is a potential risk of foot injury.

When moving materials on hand trucks or dollies, push rather than pull whenever possible.

Do not stand under loads that are suspended by ropes, chains or cables. Stand clear when ropes, cables and chains are under tension. Check the rope, cable or chain prior to use as a lifting aid.

When power equipment is being used to raise or lower materials outside of the operator’s direct view, one person in clear view of both the load and the operator shall give standard hand signals and the operator shall accept his signals only.

Stack materials/containers securely, using cross tier or pyramid methods.

Damaged containers should be isolated for evaluation prior to storage or use.

All pallet/container protruding nails must be bent over or removed as soon as possible.

Adequate lighting shall be provided for all work areas.  If in doubt about the light level, notify your supervisor for evaluation.

For rebar tying and installation, employees should make every effort to minimize working in bent over or awkward positions. Employees should also stretch hands, wrists, and back before work begins and periodically throughout the day.

All exposed rebar ends should be capped to prevent impalement and scrapes.This site uses CSS rules that your browser cannot handle, to view the page as intended please upgrade your browser. Click here for a list of recommended browsers.

MOTOR VEHICLE OPERATION

In recognition of the high risks associated with motor vehicle operation, the Company has implemented the following requirements:

  - All employees who drive Company vehicles must have a valid Oklahoma driver’s license, operator, commercial or chauffeur’s license, as required for the equipment being operated.

  - Only Company employees are permitted to operate or ride in Company owned equipment/vehicles.

  - Employees are required to obey all Oklahoma traffic regulations. Costs for violations of traffic laws are the responsibility of the employee. Employees convicted of two or more traffic moving violations within a 12-month period will not be allowed to operate Company vehicles.

  - Employees must report all vehicle accidents as required by Company and state accident reporting procedures. Employees involved in vehicle collisions will report the incident to their supervisor or designee immediately after contacting the local law enforcement agency.

Additional requirements involving motor vehicle operations are included in the Company Driver Program

HEAVY EQUIPMENT

Improper equipment operating procedures used by our employees can cause injury, disability, or death. By outlining and following safe operating procedures for use of heavy equipment, we learn to prevent injury and safeguard ourselves and our coworkers. Our goal is to ensure all employees know and understand the safe operating procedures for the safe operation and maintenance of heavy equipment.

When using heavy equipment, there are basic guidelines that employees must always follow to ensure safety:

- Know how to properly operate the equipment you are using. Supervisors shall verify that operators are capable and qualified on equipment before allowing the equipment to be operated.

- Operators shall wear seat belts and /or shoulder harnesses as provided.

- When mounting or dismounting equipment, always maintain three points of contact. Use steps and handholds provided, do not jump from the equipment.

- Inspect your equipment to ensure that it is in good working condition before beginning a job. Equipment should have seat belts, rollover protection (ROP’s), horn, and backup alarms. In addition, ensure that regular inspections and maintenance are conducted as appropriate.

- Never get on or off moving equipment or allow additional riders on trucks or other equipment unless they are designed for this purpose and you are authorized to do so.

- All employees on the ground should keep clear of heavy equipment, especially when at the rear where operators cannot see you. Do not get caught in a position where you can be struck without warning. Under no circumstances are you permitted to work under a suspended load.

For  additional requirements involving heavy equipment operation, refer to the Company Heavy Equipment Program.

SAFETY POLICY/PROCEDURES ACKNOWLEDGEMENT

To Management:

By my signature below, I acknowledge that:

- I have received and read the Company’s safety policy/procedures and I have been given the opportunity to ask questions. I have received clarification of any area of the policy/procedures I had questions about, if any, and understand the contents.  I agree to abide by the provisions in the policy/procedures and any other supplements provided.

- I have been trained for my job. I fully understand the physical requirements of my job.  I have received training in and do understand proper lifting and moving techniques, which, I am expected to use in moving or lifting objects.

- I have been informed and do fully understand that any injury claimed by me while on the job must be documented by an Accident Investigation Report filed immediately by me after my injury. I understand that unless an Accident Investigation Report is completed immediately and signed by me, the Company will doubt the validity of the injury. Every injury occurring on the job, regardless of its severity, must be documented by an Accident Investigation Report.

- I have been informed and I understand that I am not encouraged to lift or move any object by myself unless I know that I can safely lift or move the object by myself. If I believe there is no one readily available to assist me in lifting or moving, I am to wait until I can obtain assistance before moving or lifting.

_________________________________       ___________________________________

Employee’s Signature                                              Witness Signature

_________________________________      ____________________________________

Printed Name                                                            Printed Name

Dated:  ___________________________      Dated:  _____________________________